# Event day: set up the scouting server

Laminate it and tape it to the Pi's case. First-time setup: SETUP_GUIDE.md. Network detail: PI_NETWORK.md. SSD and backups: PI_STORAGE.md.

[ PHOTO: the Pi, the switch and every cable plugged in correctly, each cable labelled ]

Kit: Pi (SSD attached) · official 27 W Pi supply · unmanaged switch and its power · one flat Ethernet cable per Chromebook · USB-C-to-Ethernet adapters · gaff tape · USB stick. No Wi-Fi hotspot, router, mesh, cellular tether or any radio network for event data, ever.

## Set up (about 10 minutes)

- Cable before power. Pi eth0 → switch. Each Chromebook → switch (adapter only where needed). Tape every cable run flat.

- Power on in order: switch first, then the Pi. Wait about a minute for the green light to settle.

- The server starts by itself. systemctl status tealteam should say active (running). If not: sudo systemctl restart tealteam; journalctl -u tealteam -n 50 says why.

- Check: in the TealTeam folder, deploy/pi/network/status.sh. Every line ok; a FAIL names the fix. no uplink is fine.

- No phone tether at the event. The Pi runs without an uplink; rankings use the latest local snapshot.

- From a Chromebook: open http://tealteam.local (else http://tealteam or http://10.101.0.1).

- Lead scout: sign in, pick the event, check the rankings age. Scouting and lead work happen on the wired Chromebooks; phones only view.

- Schedule posted: on the Pi, deploy/event/prep-event.sh deploy/profiles/ftc-qual-4robot.env --roster roster.csv --csv athens.csv. Check every match against the printed schedule. If it changes, run it again: observations are kept.

## Between match blocks: back up to the stick

Plug in the stick, then sudo /opt/tealteam/tt-web backup /media/$USER/STICK. Wait for “It is on the disk.”, check the observation count looks right, eject, then pull it. (The server also snapshots to the SSD every 10 minutes; the stick is the copy that leaves the building.)

## If something is wrong

| Symptom | Do this |
| --- | --- |
| Chromebook can't open the page | Cable and adapter seated? Switch powered? Wi-Fi off? Try http://10.101.0.1. Run status.sh. |
| status.sh: eth0 FAIL | Switch unpowered or the Pi's cable is loose. |
| status.sh: app FAIL | sudo systemctl restart tealteam; still failing → is the SSD plugged in? |
| Rankings stale (amber) | No uplink by design. Use the local snapshot; note when upstream data was last updated. |
| Pi lost power | Check the Pi's clock (date) once it is back up: it has no clock battery. |
| App down for more than a match | Switch to the paper sheets. Write the time on the paper-switch log. |
| FTA asks about wireless | There is none. Offer to unplug the phone; nothing else changes. |

## Tear down

- Last backup to the stick, and eject it.

- sudo poweroff and wait for the green light to stop.

- Unplug the Pi, then the switch. Count the cables, adapters and the stick.